## Appendix A – District 24 Measurements

Table 6-12 Table of measurements made in district 24 at miscellaneous points and diversion structures through April 2020. This list does not include satellite telemetry structures, including Ventero below Sanchez after telemetry was installed and Culebra at San Luis gaging station. WDID is the structure identifier used by the Colorado Division of Water Resources within their databases.

| WDID | Station Name | Measurement Date and Time | Gage Height | Discharge | Shift (percent difference) | Measurement device |
| --- | --- | --- | --- | --- | --- | --- |
| 2400522 | CERRO D | 4/17/2012 12:43 | 0.85 | 11.5 | -0.04 | 4 FT PARSHALL |
| 2400522 | CERRO D | 5/29/2013 13:07 | 1.25 | 21.1 | -0.06 | 4 FT PARSHALL |
| 2400522 | CERRO D | 6/11/2014 11:53 | 1.54 | 30.2 | -0.04 | 4 FT PARSHALL |
| 2400527 | CHOURY D | 5/17/2012 10:13 | 0.69 | 4.97 | 0.05 | 2 FT P.F. |
| 2400532 | CULEBRA CERRITOS CANAL | 6/27/2013 13:30 | 0.87 | 9.81 | 0.01 | 6 FT PARSHALL |
| 2400533 | CULEBRA EASTDALE D | 4/17/2012 15:18 | 1.07 | 1.83 | 0.04 | RATED SECTION |
| 2400533 | CULEBRA EASTDALE D | 4/25/2012 15:43 | 1.4 | 19.8 | 0 | RATED SECTION |
| 2400533 | CULEBRA EASTDALE D | 4/4/2013 10:06 | 1.39 | 21.1 | 0.02 | RATED SECTION |
| 2400533 | CULEBRA EASTDALE D | 5/14/2013 11:17 | 1.47 | 22.9 | -0.03 | RATED SECTION |
| 2400533 | CULEBRA EASTDALE D | 5/29/2013 10:34 | 1.31 | 15.4 | 0.03 | RATED SECTION |
| 2400533 | CULEBRA EASTDALE D | 5/7/2014 11:03 | 1.36 | 20.6 | 0.05 | RATED SECTION |
| 2400533 | CULEBRA EASTDALE D | 5/20/2014 13:56 | 1.13 | 6.42 | 0.07 | RATED SECTION |
| 2400533 | CULEBRA EASTDALE D | 7/27/2015 11:47 |  |  |  | RATED SECTION |
| 2400533 | CULEBRA EASTDALE D | 5/7/2018 10:44 | 1.28 | 14.5 | 0.05 | RATED SECTION |
| 2400533 | CULEBRA EASTDALE D | 5/7/2018 10:44 | 1.28 | 14.3 |  | RATED SECTION |
| 2400533 | CULEBRA EASTDALE D | 6/24/2019 10:38 | 1.48 | 29.5 | 0.04 | RATED SECTION |
| 2400541 | ESPINOSA D | 5/17/2012 15:51 | 0 | 0.26 |  |  |
| 2400542 | FARES JAQUEZ D | 6/4/2014 13:09 | 0.35 | 1.15 | 0.18 |  |
| 2400554 | JACQUEZ D | 5/17/2012 13:47 | 0 | 2.04 |  | 9 IN PARSHALL |
| 2400554 | JACQUEZ D | 6/4/2014 9:40 |  | 0.9 |  | 9 IN PARSHALL |
| 2400554 | JACQUEZ D | 6/4/2014 10:11 |  | 1.98 |  | 9 IN PARSHALL |
| 2400554 | JACQUEZ D | 6/24/2019 12:01 |  | 1.84 |  | 9 IN PARSHALL |
| 2400555 | JAROSO D | 5/17/2012 11:06 | 0.13 | 0.57 | 0.2 (1.24%) | 1 FT PARSHALL |
| 2400574 | QUATES D 2 | 5/17/2012 12:06 | 0.35 | 1.41 | -0.1 (2.92%) | 1 FT PARSHALL |
| 2400574 | QUATES D 2 | 6/4/2014 13:54 |  | 8.85 |  | 1 FT PARSHALL |
| 2400574 | QUATES D 2 | 6/11/2014 9:58 | 0.85 | 3.48 | 0.06 (2.05%) | 1 FT PARSHALL |
| 2400579 | SALAZAR D | 5/10/2017 13:23 | 0.6 | 3.96 | -0.05 | 30" PARSHALL |
| 2400579 | SALAZAR D | 5/24/2017 13:00 | 0.71 | 5.09 | -0.06 | 30" PARSHALL |
| 2400579 | SALAZAR D | 5/24/2017 13:25 | 0.14 | 0.36 | -0.02 | 30" PARSHALL |
| 2400581 | SAN ACACIO D | 5/29/2013 11:37 | 1.23 | 23.4 | 0.04 | 4 FT PARSHALL |
| 2400581 | SAN ACACIO D | 5/7/2014 9:23 | 1.16 | 19.7 | -0.02 | 4 FT PARSHALL |
| 2400581 | SAN ACACIO D | 5/7/2014 10:15 | 0.45 | 1.94 | -0.05 | 4 FT PARSHALL |
| 2400583 | SAN FRANCISCO D | 5/17/2012 14:26 | 0.54 | 3.27 | 0.02 | N.P.F.2.'S.P.F.2* |
| 2400583 | SAN FRANCISCO D | 5/17/2012 14:50 | 0.46 | 2.46 | 0.01 | N.P.F.2.'S.P.F.2* |
| 2400583 | SAN FRANCISCO D | 5/29/2013 14:51 | 0.76 | 5.78 | 0.05 | N.P.F.2.'S.P.F.2* |
| 2400583 | SAN FRANCISCO D | 5/29/2013 15:15 | 0.6 | 4.02 | 0.04 | N.P.F.2.'S.P.F.2* |
| 2400583 | SAN FRANCISCO D | 5/7/2014 13:54 | 1 | 9.4 | 0.11 | N.P.F.2.'S.P.F.2* |
| 2400583 | SAN FRANCISCO D | 5/7/2014 14:19 | 0.62 | 4.43 | 0.06 | N.P.F.2.'S.P.F.2* |
| 2400586 | SAN LUIS PEOPLES D | 6/27/2013 9:20 |  | 12 |  | 4 FT PARSHALL |
| 2400586 | SAN LUIS PEOPLES D | 6/27/2013 10:15 | 0.84 | 10.4 | -0.08 | 4 FT PARSHALL |
| 2400586 | SAN LUIS PEOPLES D | 5/5/2015 13:24 | 0.77 | 8.72 | -0.09 | 4 FT PARSHALL |
| 2400587 | RITO SECO FEEDER | 5/10/2017 13:58 |  | 3.46 |  | 1 FT PARSHALL |
| 2400589 | SAN PEDRO D | 4/17/2012 11:47 | 0.8 | 16.2 | 0.21 | 4 FT PARSHALL |
| 2400589 | SAN PEDRO D | 5/3/2012 10:33 | 0.82 | 15.1 | 0.14 | 4 FT PARSHALL |
| 2400589 | SAN PEDRO D | 5/3/2012 11:17 | 0.69 | 12.4 | 0.16 | 4 FT PARSHALL |
| 2400589 | SAN PEDRO D | 5/3/2012 15:23 | 0.7 | 11.8 | 0.12 | 4 FT PARSHALL |
| 2400589 | SAN PEDRO D | 5/29/2013 12:23 | 1.22 | 27.5 | 0.19 | 4 FT PARSHALL |
| 2400589 | SAN PEDRO D | 5/7/2014 12:16 | 0.8 | 14.9 | 0.15 (0.68%) | 4 FT PARSHALL |
| 2400589 | SAN PEDRO D | 6/11/2014 13:03 | 1.36 | 33.4 | 0.23 | 4 FT PARSHALL |
| 2400589 | SAN PEDRO D | 5/5/2015 14:33 | 0.7 | 12.2 | 0.14 | 4 FT PARSHALL |
| 2400589 | SAN PEDRO D | 5/7/2018 12:12 | 0.89 | 17.9 | 0.18 | 4 FT PARSHALL |
| 2400589 | SAN PEDRO D | 5/8/2018 11:51 | 0.69 | 12.8 | 0.18 | 4 FT PARSHALL |
| 2400594 | VALLEJOS D | 5/3/2012 14:07 | 0.6 | 7.21 | 0.12 | N. P.F.3.'S. P.F* |
| 2400594 | VALLEJOS D | 5/29/2013 13:43 | 0.53 | 4.63 | 0.01 | N. P.F.3.'S. P.F* |
| 2400594 | VALLEJOS D | 5/29/2013 14:04 | 0.56 | 1.89 | -0.09 | N. P.F.3.'S. P.F* |
| 2400594 | VALLEJOS D | 5/7/2014 12:52 | 0.54 | 5.03 | 0.03 | N. P.F.3.'S. P.F* |
| 2400594 | VALLEJOS D | 5/7/2014 13:17 | 0.77 | 4 | 0 | N. P.F.3.'S. P.F* |
| 2400594 | VALLEJOS D | 6/11/2014 11:01 | 0.9 | 10.9 | 0.04 | N. P.F.3.'S. P.F* |
| 2400594 | VALLEJOS D | 6/11/2014 11:23 | 1.04 | 5.27 | -0.11 (-1.86%) | N. P.F.3.'S. P.F* |
| 2400594 | VALLEJOS D | 5/5/2015 15:38 | 0.51 | 4.56 | 0.33 | N. P.F.3.'S. P.F* |
| 2400594 | VALLEJOS D | 5/5/2015 16:28 | 1.02 | 5.3 | -0.1 | N. P.F.3.'S. P.F* |
| 2400594 | VALLEJOS D | 5/10/2017 14:52 |  | 4.5 |  | N. P.F.3.'S. P.F* |
| 2400599 | W F MEYER D | 5/3/2012 12:24 | 0.5 | 3.95 | -0.09 | 3 FT PARSHALL |
| 2400599 | W F MEYER D | 5/17/2012 12:54 | 0.46 | 3.61 | -0.07 | 3 FT PARSHALL |
| 2400599 | W F MEYER D | 6/4/2014 12:29 | 0.78 | 5.83 | 0.04 | 3 FT PARSHALL |
| 2400605 | CULEBRA SANCHEZ CANAL | 6/4/2014 11:02 | 1.72 | 110 | -0.01 | 12FT PARSHALL |
| 2400605 | CULEBRA SANCHEZ CANAL | 6/6/2014 13:44 | 1.46 | 94.5 | 0.09 | 12FT PARSHALL |
| 2402000 | WILLOW CK ABOVE WF MEYER DIVSERSION | 5/3/2012 12:50 | 0.4 | 4.83 | 0.01 |  |
| 2402000 | WILLOW CK ABOVE WF MEYER DIVSERSION | 5/17/2012 12:29 | 0.4 | 5.24 | 0.03 |  |
| 2402004 | VENTERO CREEK BELOW SANC | 5/31/2017 10:54 | 1.07 | 42.9 | -0.02 |  |
| 2402004 | VENTERO CREEK BELOW SANC | 5/31/2017 12:19 | 1.29 | 57.9 | -0.02 |  |
| 2402004 | VENTERO CREEK BELOW SANC | 5/31/2017 13:06 | 0.55 | 15 | 0 |  |
| 2403579 | SALAZAR RES NO 1 | 5/17/2012 15:39 | 0 | 0.92 |  | SLOPE |

*San Francisco ditch is measured at two separate Parshall flumes, the north flume and the south flume.

Flume – a specially shaped section of channel that causes flow to accelerate (USBR 2001 ed).

Parshall Flume – refers to a particularly shaped flume that is frequently used in the Culebra Basin, the size of the Parshall flume refers to the width of the throat section, shown in Figure 10-38.

Rated Section – A measurement structure that has a custom developed curve and table developed to relate measured stage to discharge.

Figure 6-38 Plan and profile view of Parshall flume (USDA-Colorado Agricultural Experiment Station 1961).