## Appendix A – Water Quality Data Conversions

Table 8-67 Sampling agency station names and locations with target group name for water quality summary.

| Station Name | Station Id | Latitude | Longitude | Group Name |
| --- | --- | --- | --- | --- |
|  | SAN-7A |  |  | Alamosito Creek |
|  | CULEBRA-GAUGE-BLANK | 37.18367 | -105.426 | Blank |
| CO0036528 Costillo County WSD-Effluent | 21COL001_WQX-CO0036528-EFF | 37.15302 | -105.409 | Costilla County WWTP |
|  | CUATES-CREEK-RD-21 | 37.02669 | -105.398 | Cuates Creek |
|  | SAN-4 |  |  | Cuates Creek |
|  | CULEBRA-CREEK-ABV-EC | 37.20579 | -105.485 | Culebra above Eastdale Canal |
|  | CULEBRA-CREEK-ABV-VTRO | 37.16341 | -105.37 | Culebra above Ventero |
|  | SAN-10 |  |  | Culebra Creek at Sanchez Canal |
| CULEBRA CREEK NEAR CHAMA, CO. | USGS-08249400 | 37.18141 | -105.321 | Culebra near Chama |
| CULEBRA CREEK AT SAN LUIS | 21COL001-8315 | 37.18389 | -105.426 | Culebra near San Luis |
| CULEBRA CREEK AT SAN LUIS | 21COL001_WQX-8315 | 37.18389 | -105.426 | Culebra near San Luis |
| Culebra Cr - At Gauge | CORIVWCH_WQX-148 | 37.18372 | -105.426 | Culebra near San Luis |
|  | CULEBRA-GAUGE | 37.18367 | -105.426 | Culebra near San Luis |
| CULEBRA CREEK AT SAN LUIS, CO | USGS-08250000 | 37.18389 | -105.426 | Culebra near San Luis |
|  | SAN-5 |  |  | Jaroso Creek - Beaver Pond |
|  | JAROSO-CREEK | 37.0546 | -105.394 | Lower Jaroso Creek |
|  | SAN-5A |  |  | Lower Jaroso Creek |
|  | SAN-6 |  |  | Lower Torcido Creek |
| RITO SECO | 21COL001-RITO SECO 01 | 37.21667 | -105.333 | Rito Seco - Wrong Lat/Long |
| RITO SECO | 21COL001-RITO SECO 02 | 37.21667 | -105.333 | Rito Seco - Wrong Lat/Long |
| RITO SECO | 21COL001-RITO SECO 03 | 37.21667 | -105.333 | Rito Seco - Wrong Lat/Long |
| RITO SECO ABV BATTLE MOUNTAIN MINE | 21COL001-8329 | 37.25416 | -105.316 | Rito Seco Above Battle Mountain Mine |
| 8329 | 21COL001_WQX-8329 | 37.25343 | -105.319 | Rito Seco Above Battle Mountain Mine |
| Rito Seco Cr - Above BMG | CORIVWCH_WQX-149 | 37.2541 | -105.315 | Rito Seco Above Battle Mountain Mine |
| RITO SECO AT SAN LUIS | 21COL001-8325 | 37.20528 | -105.424 | Rito Seco at San Luis |
| RITO SECO AT SAN LUIS | 21COL001_WQX-8325 | 37.20528 | -105.424 | Rito Seco at San Luis |
|  | CENTENNIAL | 37.20929 | -105.42 | Rito Seco at San Luis |
| RITO SECO BLW BATTLE MOUNTAIN MINE | 21COL001-8328 | 37.2465 | -105.374 | Rito Seco Below Battle Mountain Mine |
| RITO SECO BLW BATTLE MOUNTAIN MINE | 21COL001_WQX-8328 | 37.2465 | -105.374 | Rito Seco Below Battle Mountain Mine |
| Rito Seco Cr - Blw Battle Mtn Gold | CORIVWCH_WQX-156 | 37.24161 | -105.386 | Rito Seco Below Battle Mountain Mine |

Table 8-67 Sampling agency station names and locations with target group name for water quality summary.--continued

| Station Name | Station Id | Latitude | Longitude | Group Name |
| --- | --- | --- | --- | --- |
|  | SAN-7B |  |  | San Francisco Creek at Beaver Pond |
|  | SAN-FRANCISCO-CREEK | 37.11181 | -105.385 | San Francisco Creek at CR21 |
|  | SAN-7 |  |  | San Franciso Creek - Seep |
| COG589082 San Luis WSD Effluent Discharge | 21COL001_WQX-COG589082-EFF | 37.19449 | -105.436 | San Luis WWTP |
|  | SAN-8 |  |  | Sanchez Canal - Middle above San Francisco Creek |
|  | SAN-SC |  |  | Sanchez Canal above Inlet |
| SANCHEZ RESERVOIR MID-LAKE UPPER | 21COL001-8350A | 37.08288 | -105.413 | Sanchez Reservoir |
| SANCHEZ RESERVOIR MID-LAKE LOWER | 21COL001-8350B | 37.08288 | -105.413 | Sanchez Reservoir |
| SANCHEZ RESERVOIR QUADRANGLE | 21COL001-SANCHEZ | 37.08944 | -105.415 | Sanchez Reservoir |
| SANCHEZ RESERVOIR MID-LAKE UPPER | 21COL001_WQX-8350A | 37.08288 | -105.413 | Sanchez Reservoir |
| SANCHEZ RESERVOIR MID-LAKE LOWER | 21COL001_WQX-8350B | 37.08288 | -105.413 | Sanchez Reservoir |
| Sanchez Reservoir, Costilla County, CO, USA | NALMS-F193119 | 37.084 | -105.414 | Sanchez Reservoir |
|  | SAN-B |  |  | Sanchez Reservoir - Sanchez Canal Inlet |
|  | SAN-C |  |  | Sanchez Reservoir between Island and West Side |
| SANCHEZ RESERVOIR NR DAM UPPER | 21COL001-8340A | 37.11135 | -105.413 | Sanchez Reservoir near Dam |
| SANCHEZ RESERVOIR NR DAM LOWER | 21COL001-8340B | 37.11135 | -105.413 | Sanchez Reservoir near Dam |
| SANCHEZ RESERVOIR QUADRANGLE | 21COL001-SANCHEZ 2 | 37.11167 | -105.411 | Sanchez Reservoir near Dam |
| SANCHEZ RESERVOIR NR DAM UPPER | 21COL001_WQX-8340A | 37.11135 | -105.413 | Sanchez Reservoir near Dam |
| SANCHEZ RESERVOIR NR DAM LOWER | 21COL001_WQX-8340B | 37.11135 | -105.413 | Sanchez Reservoir near Dam |
| SANCHEZ RESERVOIR | 21COL001_WQX-CPW-117 | 37.11024 | -105.412 | Sanchez Reservoir near Dam |
|  | SAN-E |  |  | Sanchez Reservoir near Dam |
|  | SAN-A |  |  | Sanchez Reservoir Ventero Inlet |
| CULEBRA SNOW SITE, CO | USGS-371242105115800 | 37.21167 | -105.199 | Snow |

Table 8-67 Sampling agency station names and locations with target group name for water quality summary.--continued

| Station Name | Station Id | Latitude | Longitude | Group Name |
| --- | --- | --- | --- | --- |
|  | SAN-13 |  |  | Torcido Creek - Middle |
|  | TORCIDO-CREEK | 37.06818 | -105.351 | Torcido Creek at Mouth of Canyon |
|  | SAN-14 |  |  | Unnamed Tributary to Alamosito Creek |
|  | SAN-6A |  |  | Upper Torcido Creek |
|  | SAN-2 |  |  | UT Ventero Creek |
|  | SAN-9 |  |  | Vallejos Creek |
|  | VALLEJOS-CREEK | 37.151 | -105.401 | Vallejos Creek at Mouth |
|  | SAN-OUT |  |  | Ventero Creek - Outlet Sanchez Reservoir |
|  | SAN-11 |  |  | Ventero Creek near inlet Sanchez Reservoir |
|  | SAN-1 |  |  | Ventero Creek Pond |
| CB00207212ADC | USGS-370845105235401 | 37.15446 | -105.393 | Well |
| CB00307335BAA | USGS-371050105321501 | 37.18057 | -105.538 | Well |
| CB00307323CDB DUP1 | USGS-371148105322801 | 37.20474 | -105.534 | Well |
| DEATH VALLEY WELL | USGS-371336105415801 | 37.22668 | -105.7 | Well |
| CB00307311BDB | USGS-371400105322801 | 37.23307 | -105.536 | Well |
| CC03207222CBA | USGS-371500105253501 | 37.25001 | -105.427 | Well |
| CC0320723ADD | USGS-371504105241201 | 37.25112 | -105.404 | Well |
|  | SAN-3 |  |  | Willow Creek |
|  | WILLOW-CREEK | 37.00936 | -105.38 | Willow Creek Upper |

Table 8-68 Complete listing of all characteristics downloaded from WQP and TetraTech2008. Characteristics were converted using the factors listed in Table 8-68.

| Characteristic | Result Sample Fraction | Units |
| --- | --- | --- |
| Acidity, (H+) | Total | mg/l |
| Alkalinity | Total | mg/l CaCO3 |
| Alkalinity, Phenolphthalein (total hydroxide+1/2 carbonate) | Total | ug/l |
| Alkalinity, Phenolphthalein (total hydroxide+1/2 carbonate) | Total | mg/l |
| Alkalinity, total | Total | mg/l |
| Alkalinity, total |  | mg/l |
| Alkalinity, total | NA | mg/l |
| Aluminum | Dissolved | ug/l |
| Aluminum | Total | ug/l |
| Aluminum | Total Recovrble | ug/l |
| Ammonia | Total | mg/l |
| Ammonia and ammonium | Dissolved | mg/l as N |
| Ammonia and ammonium | Dissolved | mg/l NH4 |
| Ammonia-nitrogen | Total | mg/l |
| Arsenic | Dissolved | ug/l |
| Arsenic | Total Recoverable | ug/l |
| Arsenic | Total | ug/l |
| Arsenic | Total Recovrble | ug/l |
| Bank erosion stability (choice list) | NA | None |
| Bank vegetative stability (choice list) | NA | None |
| Bicarbonate | Total | mg/l |
| Boron | Dissolved | ug/l |
| Bromide | Total | ug/l |
| Cadmium | Dissolved | ug/l |
| Cadmium | Total | ug/l |
| Cadmium | Total Recovrble | ug/l |
| Calcium | Total | mg/l |
| Calcium | Dissolved | mg/l |
| Calcium | Total | ug/l |
| Calcium | Dissolved | ug/l |
| Carbon dioxide | Total | mg/l |
| Carbonate | Total | mg/l |
| Chloride | Total | mg/l |
| Chloride | Total | ug/l |
| Chloride | Dissolved | mg/l |
| Chlorophyll a | Non-filterable | mg/l |
| Chlorophyll a |  | ug/l |
| Chlorophyll a, corrected for pheophytin | NA | ug/l |

Table 8-68 Complete listing of all characteristics downloaded from WQP and TetraTech2008. Characteristics were converted using the factors listed in Table 8-68.--continued

| Characteristic | Result Sample Fraction | Units |
| --- | --- | --- |
| Chlorophyll a, corrected for pheophytin | None | ug/l |
| Copper | Dissolved | ug/l |
| Copper | Total | ug/l |
| Copper | Total Recovrble | ug/l |
| Count | NA | count |
| Cross-section Depth | NA | ft |
| Cyanide | Dissolved | ug/l |
| Cyanide | Total | mg/l |
| Cyanides amenable to chlorination (HCN & CN) | Total | mg/l |
| Depth, Secchi disk depth | NA | m |
| Depth, Secchi disk depth |  | m |
| Depth, snow cover | NA | in |
| Deuterium/Hydrogen ratio | Total | per mil |
| Dissolved oxygen (DO) | NA | mg/l |
| Dissolved oxygen (DO) | NA | ppm |
| Dissolved oxygen (DO) | None | mg/l |
| Dissolved oxygen (DO) |  | mg/l |
| Dissolved oxygen saturation | NA | % |
| Dissolved oxygen saturation |  | % |
| Escherichia coli | Total | #/100ml |
| Escherichia coli | Total | NA |
| Fast Riffle | NA | None |
| Fecal Coliform | Total | #/100ml |
| Flow | NA | mgd |
| Flow | NA | ft3/sec |
| Flow |  | cfs |
| Flow | NA | cfs |
| Fluoride | Dissolved | mg/l |
| Hardness, Ca, Mg | Total | mg/l |
| Hardness, Ca, Mg | NA | mg/l CaCO3 |
| Hardness, Ca, Mg | NA | mg/l |
| Hardness, non-carbonate | Total | mg/l CaCO3 |
| Inorganic nitrogen (ammonia, nitrate and nitrite) | Total | mg/l |
| Inorganic nitrogen (nitrate and nitrite) | Total | mg/l |
| Inorganic nitrogen (nitrate and nitrite) | Dissolved | mg/l as N |
| Inorganic nitrogen (nitrate and nitrite) as N | Total | mg/l |
| Iron | Dissolved | ug/l |
| Iron | Total Recovrble | ug/l |

Table 8-68 Complete listing of all characteristics downloaded from WQP and TetraTech2008. Characteristics were converted using the factors listed in Table 8-68.--continued

| Characteristic | Result Sample Fraction | Units |
| --- | --- | --- |
| Iron | Total Recoverable | ug/l |
| Iron | Total | ug/l |
| Kick Depth | NA | ft |
| Kick Time | NA | seconds |
| Kjeldahl nitrogen | Total | mg/l |
| Lead | Dissolved | ug/l |
| Lead | Total | ug/l |
| Lead | Total Recovrble | ug/l |
| Magnesium | Total | mg/l |
| Magnesium | Dissolved | mg/l |
| Magnesium | Total | ug/l |
| Magnesium | Dissolved | ug/l |
| Manganese | Dissolved | ug/l |
| Manganese | Total | ug/l |
| Manganese | Total Recovrble | ug/l |
| Mercury | Total | ug/L |
| Mercury | Dissolved | ug/L |
| Mercury | Total | ug/l |
| Mercury | Dissolved | ug/l |
| Mercury | Total | ng/l |
| Methylmercury | Total | ug/L |
| Methylmercury | Dissolved | ug/L |
| Nitrate | Total | mg/l |
| Nitrate | Dissolved | mg/l as N |
| Nitrate | Dissolved | mg/l asNO3 |
| Nitrate as N | Total | mg/l |
| Nitrite | Total | mg/l |
| Nitrite as N | Total | mg/l |
| Nitrogen, mixed forms (NH3), (NH4), organic, (NO2) and (NO3) | Total | mg/l |
| Organic carbon | Dissolved | mg/l |
| Orthophosphate | Dissolved | mg/l asPO4 |
| Oxygen-18/Oxygen-16 ratio | Total | per mil |
| pH | Total | None |
| pH | NA | None |
| pH |  | None |
| pH | None | None |
| pH | Total | std units |
| Phosphate-phosphorus | Total | mg/l |

Table 8-68 Complete listing of all characteristics downloaded from WQP and TetraTech2008. Characteristics were converted using the factors listed in Table 8-68.--continued

| Characteristic | Result Sample Fraction | Units |
| --- | --- | --- |
| Phosphate-phosphorus as P | Total | mg/l |
| Phosphorus | Total | mg/l |
| Phosphorus | Dissolved | mg/l as P |
| Potassium | Total | mg/l |
| Potassium | Dissolved | mg/l |
| Potassium | Total | ug/l |
| Potassium | Dissolved | ug/l |
| RBP2, Aquatic Vegetation, Dominant Type & Species (choice list) | NA | None |
| RBP2, Aquatic vegetation, portion of reach with AV (%) | NA | % |
| RBP2, Habitat type, gravel-cobble (%) | NA | % |
| RBP2, Habitat type, pools (%) | NA | % |
| RBP2, Habitat type, riffle (%) | NA | % |
| RBP2, Habitat type, run (%) | NA | % |
| RBP2, Habitat type, sand (%) | NA | % |
| RBP2, Habitat type, snags (%) | NA | % |
| RBP2, Habitat type, vegetated banks (%) | NA | % |
| RBP2, Instream Features, Canopy Cover (choice list) | NA | None |
| RBP2, Instream Features, Channelized (Y/N) (choice list) | NA | None |
| RBP2, Instream features, est. stream depth | NA | ft |
| RBP2, Instream features, est. stream width | NA | ft |
| RBP2, Instream features, high water mark | NA | ft |
| RBP2, Riparian Vegetation, Dominant Species Present (choice list) | NA | None |
| RBP2, Substrate, inorganic, bedrock | NA | % |
| RBP2, Substrate, Inorganic, boulder, >256 mm | NA | % |
| RBP2, Substrate, inorganic, clay, <0.004 mm | NA | % |
| RBP2, Substrate, inorganic, cobble, 64-256 mm | NA | % |
| RBP2, Substrate, inorganic, gravel, 2-64 mm | NA | % |
| RBP2, Substrate, inorganic, sand, 0.06-2.0 mm | NA | % |
| RBP2, Substrate, Inorganic, Silt, 0.004-0.06 mm | NA | % |
| RBP2, Substrate, organic, detritus, sticks, wood, etc. (CPOM) | NA | % |
| RBP2, Substrate, organic, marl, grey shell fragments | NA | % |
| RBP2, Substrate, organic, muck-mud, black-fine (FPOM) | NA | % |
| RBP2, Watershed, Local Erosion (choice list) | NA | None |
| RBP2, Watershed, Predominant Surrounding Land use (choice list) | NA | None |
| Riparian Width LDB | NA | ft |
| Riparian Width RDB | NA | ft |
| Rocky Substrate (Choice List) | NA | None |
| Sand/Substrate Habitat Type (%) | NA | % |

Table 8-68 Complete listing of all characteristics downloaded from WQP and TetraTech2008. Characteristics were converted using the factors listed in Table 8-68.--continued

| Characteristic | Result Sample Fraction | Units |
| --- | --- | --- |
| Sandy Substrate (Choice List) | NA | None |
| Selenium | Dissolved | ug/l |
| Selenium | Total | ug/l |
| Selenium | Total Recovrble | ug/l |
| Silica | Dissolved | mg/l |
| Silver | Dissolved | ug/l |
| Silver | Total Recovrble | ug/l |
| Slow Riffle | NA | None |
| Sodium | Total | mg/l |
| Sodium | Dissolved | mg/l |
| Sodium | Total | ug/l |
| Sodium | Dissolved | ug/l |
| Sodium adsorption ratio [(Na)/(sq root of 1/2 Ca + Mg)] | NA | None |
| Sodium, percent total cations | NA | % |
| Specific conductance | Total | uS/cm |
| Specific conductance | NA | uS/cm |
| Specific conductance |  | uS/cm |
| Specific conductance | None | uS/cm |
| Specific conductance | Total | uS/cm @25C |
| Specific conductance |  | umho/cm |
| Specific conductance | NA | umho/cm |
| Stream flow, instantaneous | NA | ft3/s |
| Stream flow, instantaneous | NA | m3/sec |
| Stream flow, mean. daily | NA | ft3/s |
| Strontium | Dissolved | ug/l |
| Submerged Vegetation Habitat Type (%) | NA | % |
| Sulfate | Total | mg/l |
| Sulfate | Dissolved | mg/l |
| Sulfate as SO4 | Total | mg/l |
| Sulfate as SO4 | Dissolved | mg/l |
| Sulfide | Total | mg/l |
| Sulfur-34/Sulfur-32 ratio | Total | per mil |
| Temperature, air |  | deg F |
| Temperature, air |  | deg C |
| Temperature, air | NA | deg C |
| Temperature, air | NA | deg F |
| Temperature, sample | NA | deg C |

Table 8-68 Complete listing of all characteristics downloaded from WQP and TetraTech2008. Characteristics were converted using the factors listed in Table 8-68.--continued

| Characteristic | Result Sample Fraction | Units |
| --- | --- | --- |
| Temperature, water | NA | deg C |
| Temperature, water | None | deg C |
| Temperature, water |  | deg C |
| Total dissolved solids | Filterable | mg/l |
| Total dissolved solids | Dissolved | mg/l |
| Total dissolved solids | Dissolved | tons/ac ft |
| Total dissolved solids | Dissolved | tons/day |
| Total Phosphorus, mixed forms | Total | mg/l |
| Total suspended solids | Non-filterable | mg/l |
| Total suspended solids | Total | mg/l |
| Uranium | Dissolved | ug/l |
| Uranium-238 | Dissolved | ug/l |
| Water Column Habitat Type (%) | NA | % |
| Water content of snow | NA | in |
| Zinc | Dissolved | ug/l |
| Zinc | Total | ug/l |
| Zinc | Total Recovrble | ug/l |

Conversions of units and naming conventions were needed prior to analyzing the existing water quality datasets. These conversions were performed using a python script that evaluated each characteristic, result sample fraction text, and unit and if needed converted the values using the mappings listed in Table 8-68.In addition to the conversions shown below all records that reported a “Result Detection Condition Text” of ‘Not Reported’ were deleted (82 records). Where ResultDetectionConditionText was ‘Non-detect’ or ‘Not Detected’ the MeasureQualifierCode was set to ‘U’. The result was set to null wherever the MeasureQalifierCode was ‘U’ to prevent results reported as <Detection Limit or equal to Detection Limit from being interpreted as values.

Table 8-69 Characteristics conversions used to align data in existing water quality datasets.

| Characteristic | Result Sample Fraction Text | Unit | Conversion Factor | Destination Characteristic | Destination Result Sample Fraction Text | Destination Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Alkalinity |  | mg/L | 1 | Alkalinity | Total | mg/l |
| Alkalinity | Total | mg/l | 1 | Alkalinity | Total | mg/l CaCO3 |
| Alkalinity, Phenolphthalein (total hydroxide+1/2 carbonate) | Total | ug/l | 0.001 | Alkalinity, Phenolphthalein (total hydroxide+1/2 carbonate) | Total | mg/l |
| Alkalinity, total |  | mg/l | 1 | Alkalinity | Total | mg/l CaCO3 |
| Alkalinity, total | NA | mg/l | 1 | Alkalinity | Total | mg/l CaCO3 |
| Alkalinity, total |  | mg/l | 1 | Alkalinity | Total | mg/l CaCO3 |
| Alkalinity, total | Total | mg/l | 1 | Alkalinity | Total | mg/l CaCO3 |
| Aluminum | Total Recovrble | ug/l | 1 | Aluminum | Total | ug/l |
| Aluminum | Dissolved | mg/L | 1000 | Aluminum | Dissolved | ug/l |
| Aluminum | Dissolved | ug/L | 1 | Aluminum | Dissolved | ug/l |
| Aluminum | Dissolved | mg/l | 1000 | Aluminum | Dissolved | ug/l |
| Ammonia-nitrogen | Total | mg/l | 1 | Ammonia | Total | mg/l |
| Antimony | Dissolved | mg/l | 1000 | Antimony | Dissolved | ug/l |
| Arsenic | Total Recoverable | ug/l | 1 | Arsenic | Total | ug/l |
| Arsenic | Total Recovrble | ug/l | 1 | Arsenic | Total | ug/l |
| Arsenic | Total | mg/l | 1000 | Arsenic | Dissolved | ug/l |
| Arsenic | Dissolved | mg/l | 1000 | Arsenic | Dissolved | ug/l |
| Barium | Dissolved | mg/l | 1000 | Barium | Dissolved | ug/l |
| Beryllium | Dissolved | mg/l | 1000 | Beryllium | Dissolved | ug/l |
| Boron | Dissolved | mg/l | 1000 | Boron | Dissolved | ug/l |
| Cadmium | Total Recovrble | ug/l | 1 | Cadmium | Total | ug/l |
| Cadmium | Dissolved | mg/l | 1000 | Cadmium | Dissolved | ug/l |
| Calcium | Total | ug/l | 0.001 | Calcium | Total | mg/l |
| Calcium | Dissolved | ug/l | 0.001 | Calcium | Dissolved | mg/l |
| Chloride | Total | ug/l | 0.001 | Chloride | Total | mg/l |
| Chromium | Dissolved | mg/l | 1000 | Chromium | Dissolved | ug/l |
| Cobalt | Dissolved | mg/l | 1000 | Cobalt | Dissolved | ug/l |
| Copper | Total Recovrble | ug/l | 1 | Copper | Total | ug/l |
| Copper | Dissolved | mg/l | 1000 | Copper | Dissolved | ug/l |

Table 8-69 Characteristics conversions used to align data in existing water quality datasets.--continued

| Characteristic | Result Sample Fraction Text | Unit | Conversion Factor | Destination Characteristic | Destination Result Sample Fraction Text | Destination Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Depth, Secchi disk depth |  | m | 1 | Depth, Secchi disk depth | NA | m |
| Dissolved oxygen (DO) | None | mg/l | 1 | Dissolved oxygen (DO) | NA | mg/l |
| Dissolved oxygen (DO) |  | mg/l | 1 | Dissolved oxygen (DO) | NA | mg/l |
| Dissolved oxygen (DO) |  | mg/l | 1 | Dissolved oxygen (DO) | NA | mg/l |
| Dissolved oxygen saturation |  | % | 1 | Dissolved oxygen saturation | NA | % |
| Dissolved oxygen saturation |  | % | 1 | Dissolved oxygen saturation | NA | % |
| Hardness, Ca, Mg | NA | mg/l | 1 | Hardness, Ca, Mg | Total | mg/l |
| Iron | Total Recovrble | ug/l | 1 | Iron | Total | ug/l |
| Iron | Total Recoverable | ug/l | 1 | Iron | Total | ug/l |
| Iron | Total | mg/l | 1000 | Iron | Total | ug/l |
| Iron | Dissolved | mg/l | 1000 | Iron | Dissolved | ug/l |
| Lead | Total Recovrble | ug/l | 1 | Lead | Total | ug/l |
| Lead | Dissolved | mg/l | 1000 | Lead | Dissolved | ug/l |
| Magnesium | Total | ug/l | 0.001 | Magnesium | Total | mg/l |
| Magnesium | Dissolved | ug/l | 0.001 | Magnesium | Dissolved | mg/l |
| Manganese | Total Recovrble | ug/l | 1 | Manganese | Total | ug/l |
| Manganese | Dissolved | mg/l | 1000 | Manganese | Dissolved | ug/l |
| Mercury | Total | ng/l | 0.001 | Mercury | Total | ug/l |
| Mercury | Total | ug/L | 1 | Mercury | Total | ug/l |
| Mercury | Dissolved | ug/L | 1 | Mercury | Dissolved | ug/l |
| Mercury | Dissolved | mg/l | 1000 | Mercury | Dissolved | ug/l |
| Methylmercury | Total | ug/L | 1 | Methylmercury | Total | ug/l |
| Methylmercury | Dissolved | ug/L | 1 | Methylmercury | Dissolved | ug/l |
| Molybdenum | Dissolved | mg/l | 1000 | Molybdenum | Dissolved | ug/l |
| Nickel | Dissolved | mg/l | 1000 | Nickel | Dissolved | ug/l |
| Nickel | Dissolved | mg/l | 1000 | Nickel | Dissolved | ug/l |
| Potassium | Total | ug/l | 0.001 | Potassium | Total | mg/l |
| Potassium | Dissolved | ug/l | 0.001 | Potassium | Dissolved | mg/l |
| Selenium | Total Recovrble | ug/l | 1 | Selenium | Total | ug/l |
| Selenium | Dissolved | mg/l | 1000 | Selenium | Dissolved | ug/l |
| Selenium | Total | mg/l | 1000 | Selenium | Total | ug/l |
| Silver | Dissolved | mg/l | 1000 | Silver | Dissolved | ug/l |

Table 8-69 Characteristics conversions used to align data in existing water quality datasets.--continued

| Characteristic | Result Sample Fraction Text | Unit | Conversion Factor | Destination Characteristic | Destination Result Sample Fraction Text | Destination Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Silver | Total Recovrble | ug/l | 1 | Silver | Total | ug/l |
| Sodium | Total | ug/l | 0.001 | Sodium | Total | mg/l |
| Sodium | Dissolved | ug/l | 0.001 | Sodium | Dissolved | mg/l |
| Specific conductance | NA | uS/cm | 1 | Specific conductance | Total | uS/cm |
| Specific conductance |  | uS/cm | 1 | Specific conductance | Total | uS/cm |
| Specific conductance | None | uS/cm | 1 | Specific conductance | Total | uS/cm |
| Specific conductance | Total | uS/cm @25C | 1 | Specific conductance | Total | uS/cm |
| Specific conductance |  | umho/cm | 1 | Specific conductance | Total | uS/cm |
| Specific conductance | NA | umho/cm | 1 | Specific conductance | Total | uS/cm |
| Specific conductance |  | uS/cm | 1 | Specific conductance | Total | uS/cm |
| Specific conductance |  | umho/cm | 1 | Specific conductance | Total | uS/cm |
| Strontium | Dissolved | mg/l | 1000 | Strontium | Dissolved | ug/l |
| Sulfate | Dissolved | mg/l | 1 | Sulfate |  | mg/l |
| Sulfate | Total | mg/l | 1 | Sulfate |  | mg/l |
| Sulfate as SO4 | Total | mg/l | 1 | Sulfate |  | mg/l |
| Sulfate as SO4 | Dissolved | mg/l | 1 | Sulfate |  | mg/l |
| Temperature, water | None | deg C | 1 | Temperature, water | NA | deg C |
| Temperature, water |  | deg C | 1 | Temperature, water | NA | deg C |
| Thallium | Dissolved | mg/l | 1000 | Thallium | Dissolved | ug/l |
| Tin | Dissolved | mg/l | 1000 | Tin | Dissolved | ug/l |
| Total dissolved solids | Filterable | mg/l | 1 | Total dissolved solids | Dissolved | mg/l |
| Vanadium | Dissolved | mg/l | 1000 | Vanadium | Dissolved | ug/l |
| Zinc | Total Recovrble | ug/l | 1 | Zinc | Total | ug/l |
| Zinc | Dissolved | mg/l | 1000 | Zinc | Dissolved | ug/l |
| pH | Total | None | 1 | pH | Total | std units |
| pH | NA | None | 1 | pH | Total | std units |
| pH |  | None | 1 | pH | Total | std units |
| pH | None | None | 1 | pH | Total | std units |
| pH |  | None | 1 | pH | Total | std units |